MATERIAL SAFETY DATA SHEET

(MSDS)



1.  PRODUCT AND COMPANY IDENTIFICATION



PRODUCT NAME:  BITUSOL 4017                                              UPDATED:  28 OCTOBER 1998

SUPPLIER:  DALVEN PRODUCTS                                              CAS NUMBER:  MIXTURE  -  NA

P.O. BOX 371, EPPINDUST 			                     UN NUMBER:  UN 1993



HEALTH EMERGENCY TELEPHONE:  (021) 689-5227  (RED CROSS POISON SERVICE)

TRANSPORT EMERGENCY TELEPHONE:  (011) 331-2947  (HAZCHEM)



2.  COMPOSITION/INFORMATION ON INGREDIENTS



CHEMICAL NAMES AND SYNONYMS:     PETROLEUM BITUMEN AND SOLVENT BLEND



INGREDIENTS CONSIDERED HAZARDOUS TO HEALTH:



Substance Name                                                                 	Wt%



OXIDIZED PETROLEUM ASPHALT           		
	
64

(64742-96-4)

HEAVY ALIPHATIC PETROLEUM                    		18

SOLVENT ( 64742-96-7)

TOLUENE                                                                           
	
18

(108-88--3)

BENZENE(71-43-2)                                                          
	
( 0.01



See Section 15 for European Label Information.



See Section  8  for exposure limits (if applicable).



3.  HAZARDS INDENTIFICATION



US OSHA HAZARD COMMUNICATION STANDARD:  Product assessed in accordance with  OSHA 29 CFR 1910.1200  and determined to be hazardous.

EFFECTS OF OVEREXPOSURE:  Skin irritation can occur under conditions of poor personal hygiene.  Respiratory  irritation, dizziness, nausea and even loss of consciousness can occur after excessive inhalation of vapours.

EMERGENCY RESPONSE DATA:  Black Semi-solid Liquid.  Material is flammable DOT ERG No.  - 128

See Section 11 for additional Biological hazards (if applicable).



4.  FIRST AID  MEASURES



EYE CONTACT:  Flush  thoroughly with water.  If irritation occurs, call a doctor.

SKIN CONTACT:  Wash contact areas with suitable  hand-cleaner.  Remove contaminated clothing.  Launder contaminated clothing before reuse.

INHALATION:  Remove from further exposure.  If respiratory irritation, dizziness, nausea, or unconsciousness occurs, seek immediate medical assistance and call a doctor.  If breathing has stopped, use mouth to mouth resuscitation.

INGESTION:   Not expected to occur in normal industrial use.  However do not  induce vomiting.  Give 1 to 2 glasses of water.  Get medical  assistance.

NOTE  TO DOCTORS:  Material if aspirated into the lungs may cause chemical pneumonitis.  Treat appropriately.



5.  FIRE-FIGHTING MEASURES



EXTINGUISHING MEDIA:  Carbon dioxide, foam, dry chemical and water fog.

SPECIAL FIRE FIGHTING PROCEDURES:  Use water to keep fire exposed containers cool.  If a leak or spill has not ignited, use water spray to disperse the vapours and to protect personnel attempting to stop leak. Water spray may be used to flush spills away from exposures.  Prevent runoff from fire control or dilution from entering streams, sewers, or drinking water supply.

SPECIAL PROTECTIVE EQUIPMENT:  For fires in enclosed areas, fire fighters must use self-contained breathing apparatus.

UNUSUAL FIRE AND EXPOLSION HAZARDS:  Material is Flammable and if exposed to flames can generate toxic fumes.



Flash Point C:  O (ASTM D-92).                                              Flammable limits  -- LEL:  NE

UEL:  NE

NFPA HAZARD ID:  Health:  O,  Flammability:  2,  Reactivity:  O

HAZARDOUS  DECOMPOSITION PRODUCTS:  Carbon monoxide.



6.  ACCIDENTAL RELEASE MEASURES



NOTIFICATION PROCEDURES:  Report spills as requires to appropriate  authorities.

PROCEDURES IF MATERIAL IS RELEASED OR SPILLED:  Adsorb on fire retardant treated sawdust, diatomaceous earth, etc.  Shovel up and dipose of at an appropriate waste disposal facility in accordance with current applicable laws and regulations, and product characteristics at time of disposal.

ENVIRONMENTAL PRECAUTIONS:  Prevent spills from entering storm sewers or drains and contact with soil.

PERSONAL PRECATIONS:  See Section 8



7.  HANDLING AND STORAGE



HANDLING:  Avoid contact with skin.  Trace amounts of Hydrogen Sulphide may be present.  Keep face clear of drum, tank and/or tank car opening.

STORAGE:  Store in a  cool area.



8.  EXPOSURE CONTROLS/PERSONAL PROTECTION



VENTILATION:  Use in well ventilated area.  Ventilation desirable and equipment should be explosion proof.

RESPIRATORY PROTECTION:  Approved respiratory  protective equipment must be used when vapour or mists concentrations exceed applicable standards.

EYE PROTECTION:  Normal industrial eye protection practices should be employed.

SKIN PROTECTION:  Impervious gloves should be worn.





















OCCUPATIONAL EXPOSURE LIMITS

	                                                                    LTEL                                       STEL            NOTES

Substance Name (CAS-No)                      Source        ppm  mg/m3                                             ppm  mg/m3                                      



OXIDIZED PETROLEUM  ASPHALT

(64742-93-4)                                            ACGIH                   5

Fumes

HEAVY ALIPHATIC PETROLEUM

SOLVENT (64742-96-7)                         ACGIH        100    525



BENZENE (71-43-2)                                 OSHA           1                                           5

                                                                 ACGIH        10      32



TOLUENE (108-88-3)              

                                                                OSHA          100   375                              150         560

Skin                                                        ACGIH           50   188  



NOTE:  Limits shown for guidance only.  Follow applicable regulations.



THRESHOLD LIMIT VALUE:  50.00 ppm



9.  PHYSICAL AND CHEMICAL PROPERTIES



Typical  physical properties are given below.  Consult Product Data Sheet

for specific details.



APPEARANCE:                                                       Semi-solid liquid

COLOUR:                                                                Black

ODOUR:                                                                  Asphaltic

ODOUR THRESHOLD-ppm:                                   NA

pH:                                                                           NA

BOILING POINTN C:                                              ( 190

MELTING   POINT C:                                              NA

FLASH POINT C:                                                     O (ASTM D-92

FLAMMABILITY:                                                   NA

AUTO FLAMMABILITY:                                       NE

EXPLOSIVE PROPERTIES:                                    NA

OXIDIZING PROPERTIES:                                     NA

VAPOUR PRESSURE-mmHg 20 C:                        ( 0.1

VAPOUR DESNITY:                                               NA

EVAPORATION RATE:                                         NA

RELATIVE DENSITY, 20 C, kg/1:                         0.905

SOLUBILITY IN WATER:                                      Negligible

PARTITION COEFFICIENT:                                  NE

VISCOSITY AT 60 C, cSt:                                      40-100

VISCOSITY AT 100 C, cSt:                                    NE

POUR POINT C:                                                     NA

FREEZING POINT C:                                             NE

VOLATILE ORGANIC COMPOUND:                   NE



NA=NOT APPLICABLE         NE=NOT  ESTABLISHED       D=DECOMPOSES



FOR FURTHER TECHNICAL INFORMATION, CONTACT YOUR MARKETING REPRESENTATIVE



10.  STABILITY AND REACTIVITY



STABILITY (THERMAL, LIGHT, ETC.):  Stable.

CONDITIONS TO AVOID:  Heat, sparks, flame and build up of static electricity.

INCOMPATIBILITY (MATERIALS TO AVOID):  Strong  oxidizers.

HAZARDOUS DECOMPOSITION PRODUCTS:  Carbon monoxide.

HAZARDOUS POLYMERIZATION:  Will not occur.



11  TOXICOLOGICAL DATA



---------ACUTE TOXICOLOGY--------



ORAL TOXICITY (RATS):  Practically non-toxic  (LD50:  greater than 2000 mg/kg).  ---Based on testing of similar products and/or the components.

DERMAL TOXICITY (RABBITS):  Practically non-toxic  (LD50:  greater than 2000 mg/kg).  ---Based on testing of similar products and/or the components.

INHALATION  TOXICITY (RATS):  Practically non-toxic (LC50:   greater than 5 mg/1).  ---Based on testing of similar products and/or the components.

EYE IRRITATION(RABBITS):  Practically non-irritating.  (Draize  score:  greater than 6 but 15 or less).  ---Based on testing of similar products and/or the components.

SKIN IRRITATION(RABBITS):  Irritant.  (Primary irritation  Index:  3 or  greater but less than 5).  ---Based on testing of similar products  and/or the  components.

OTHER ACUTE TOXICITY DATA:  Inhalation of vapours/mists may cause respiratory system irritation.





------GENETIC TOXICOLOGY (SUMMARY)------



Mutagenicity studies with similar materials in the ames, mouse lymphoma, rat bone marrow cytogenetics and dominant lethal assays showed no adverse effects.



-------REPRODUCTIVE TOXICOLOGY (SUMMARY)---



No birth defects were reported when  pregnant rats, mice or rabbits were exposed to vapours at levels up to 1000 ppm of toluene.  There were some toxic effects on developing foetuses in other studies including decreased body weight and survival at 267 ppm or greater.  A  2-generation reproduction study with rats did not result in birth defects at levels up to 2000 ppm, but there was some growth inhibition.  The no-effect level (NOEL)  for toluene was 500 ppm.



---NEUROTOXICOLOGY (SUMMARY)--



Toluene  Inhalation studies (1500 ppm) , 6 hours/day, 5 days/week for 6 months in rats:  No neurotoxicity.



---CHRONIC TOXICOLOGY (SUMMARY)---



No carcinogenic effects were observed when toluene was applied to the skin of mice for two years.  A 2 year NTP inhalation study at levels up to 1200 ppm toluene showed no carcinogenic efects in rats or mice.  Prolonged or related exposure to  toluene does not  result in bone marrow injury or blood changes characteristic of benzene.  Chronic exposure to hight concentrations of benzene have been associated with blood abnormalities and leukemia in a small percentage of exposed workers.  NOTE  This product is specified to contain less than 0.01 % m/m  benzene.

This product contains solvents, prolonged repeated skin contact may  defat the skin resulting in possible irritation and dermatitis.



12.  ECOLOGICAL INFORMATION



ENVIRONMENTAL FATE AND EFFECTS:



Acute LC/EC50 Fish:  Juvenile Rainbow Trout:  Harmful---Based on testing of similar products.



13.  DISPOSAL CONSIDERATIONS



WASTE DISPOSAL:  Product is suitable for burning in an enclosed controlled burner for fuel value or disposal  by supervised incineration.



RCRA INFORMATION:  The unused product, in our opinion, is not specifically listed by the EPA as a hazardous waste  (40 CFR, Part 261D, nor is it formulated to contain mateials which are listed hazardous wastes.  It does not exhibit the hazardous characteristics of ignitability, corrosivity, or reactivity and is not formulated with contaminats as determined by the  Toxicity Characteristic :Leaching Procedure (TCLP).  However, used product may be  regulated.



14.  TRANSPORT  INFORMATION



USA DOT:                       

	SHIPPING NAME:               		Flammable liquid, n.o.s.(contains 									ASPHALT, TOLUENE and APHATIC 								PETROLEUM SOLVENT)	

	HAZARD CLASS & DIV:                            Flammable liquid

	ID NUMBER:                                               NA1993

	ERG NUMBER:                                           128

	PACKAGING GROUP:                                PG III

	STCC:                                                          NE

	DANGEROUS WHEN WET       		NO

	POISON:                                                      NO

	LABEL(s):                                                    NA

	PLACARD(s):                                              Flammable

	PRODUCT RQ:                                            NA

	MARPOL III STAT  :                                   NA



IMO:

	HAZARD CLASS & DIV: 		3.3

	UN NUMBER:                                             1993

	PACKAGING GROUP:                               PG III

	SHIPPING NAME:                                      Flammable liquid, n.o.s. (contains ASPHALT,

                                                                                  TOLUENE and ALIPHATIC PETROLEUM     

                                                                                  SOLVENT)

	LABEL(s):                                                   Flammable liquid

	MARPOL III STATUS:                               NA



ICAO/IATA:

	HAZARD CLASS & DIV:      		 3

	ID/UN NUMBER:                                        1993

	PACKING GROUP:                                     PG III

	SHIPPING NAME:                                      Flammable liquids n.o.s. (contains ALPHALT

                                                                                  TOLUENE and ALIPHATIC   PETROLEUM                                   						SOLVENT)

	SUBSIDIARY RISK:                                   NA

	LABEL(s):                                                                                                       Flammable  Liquid





15.  REGULATORY INFORMATION



GOVERNMENTAL INVENTORY STATUS:  All components comply with TSCA, and EINECS/ELINCS.



EU Labelling:

	Symbol:  F Flammable.



	Risk  Phrase(s):  R10-20-38-52/53.

	Flammable.  Harmful by inhalation.  Irritating to skin.   Harmful to aquataic organisms, may 	cause long-term adverse effects in the aquatic environment.



Safety  Phrase(s):  S36/37-43-61-62.

	Wear suitable protective clothing and gloves.  In case of fire use foam/drypowder/CO2/ Halon.  	Avoid release to the environment.  Refer to special instructions/Safety data sheets.  If  swallowed, 	do not induce vomiting:  seek medical advice immediately and show this container or label.



Contains:  Petroleum distillate, toluene and Benzene. ((.01%)

.

U.S.  Superfund Amendments and Reauthorization Act (SARA)  Title III :  

This product contains no “EXTREMELY HAZARDOUS SUBSTANCES”.



SARA (311/312)  REPORTABLE HAZARD CATEGORIES:

FIRE  CHRONIC  ACUTE



This  product contains the following SARA (313) Toxic Release Chemicals:



CHEMICAL NAME                       	CAS NUMBER                                        CONC.



BENZENE (COMPONENT ANALYSIS)     71-43-2                                                    ( 0.01%

TOLUENE    		                           108-88-3                                                       18%





U.S.  Drug Enforcement Administration Chemical Diversion and Trafficking Act:  Sales, receipts, movements or unexplainable losses of the following chemical(s) may require recordkeeping and reporting in accordance with 21 CFR 1310/1313  ***  TOLUENE





The  following product ingredients are cited on the lists below:



CHEMICAL NAME                                 		CAS NUMBER  		LIST CITATIONS



HEAVY ALIPHATIC PETROLEUM SOLVENT     64742-96-7                        1, 18, 19, 20, 21

                                                                                                                           23, 25

TOLUENE (100.00%)                                              108-88-3                            1, 10, 17, 18, 19, 

                                                                                                                           20, 21, 22, 23, 24



BENZENE (COMPONENT ANALYSIS)                71-43-2                              1, 3, 4, 6, 9, 17,

(( 0.01%)                                                                                                           22, 24,25







-----REGULATORY LISTS SEARCHED----



1=ACGIH ALL        6=IARC 1             11=TSCA 4               17=CA P65                     22=MI 293

2=ACGIH A1           7=IARC  2A         12=TSCA 5a2            18=CA RTK                   23=MN  RTK

3=ACGIH A2           8=IARC  2B         13=TSCA 5e              19=FL RTK                    24=NJ  RTK

4=NTP  CARC         9=OSHA  CARC  14=TSCA 6                20=IL RTK                     25=PA RTK

5=NTP  SUS            10= OSHA  Z        15=TSCA  12b           21=LA  RTK                  26=RI  RTK

 



Code key:      CARC=Carcinogen;                SUS=Suspected Carcinogen





16.  OTHER INFORMATION



USE:  ADHESIVE, SEALANT AND WATERPROOFING AGENT.



INGREDIENT DESCRIPTION                                		PERCENT                        CAS NUMBER





OXIDIZED PETROLEUM ASPHALT                                    64                                         64742-93-4

HEAVY  ALIPHATIC  PEROLEUM  SOLVENT  		18                                         64742-96-7

TOLUENE                                                                              18                                        108-88-3





INFORMATION GIVEN HEREIN IS OFFERED IN GOOD FATIH AS ACCURATE, BUT WITHOUT GUARANTEE.  CONDITIONS OF USE AND SUITABILITY OFTHE PRODUCT FOR PARTICULAR USES  ARE BEYOND OUR CONTROL; ALL RISKS OF  USE OF THE  PRODUCT  ARE THEREFORE ASSUMED BY THE USER AND WE EXPRESSLY DISCLAIM ALL WARRANTIES OF EVERY KIND AND NATURE, INCLUDING WARRANTIES OF MERCHANTABILITY AND FITNESS  FOR A PARTICULAR PURPOSE IN RESPECT TO THE USE OR SUITABILITY OF THE PRODUCT.  NOTHING IS INTENDED AS A RECOMMENDATION FOR USES WHICH INFRINGE VALID PATENTS OR AS EXTENDING  LICENSE UNDER VALID PATENTS.  APPROPRIATE WARNINGS AND SAFE HANDLING PROCEDURES SHOULD BE PROVIDED TO HANDLERS AND USERS.



Prepared for:  DALVEN PRODUCTS                  

                           P.O. BOX 371

                           EPPINDUST

		7475

For  further information, contact:	Product Safety Adviser :  (021)  403 4805;	  Cell   083 628 4415                               













�PAGE  �






�PAGE  �
1
�

BITUSOL 4017






�PAGE  �

















