MATERIAL SAFETY DATA SHEETS


(MSDS)





1.  PRODUCT AND COMPANY IDENTIFICATION





                                                                                                                  UPDATED:  26 OCTOBER 1998


PRODUCT NAME:  BITUSOL 4098                                                    CAS NUMBER:  MIXTURE - NA


SUPPLIER:  DALVEN PRODUCTS                                                     UN NUMBER:  UN1993


                      P.O. BOX 371


                      EPPINDUST


HEALTHY EMERGENCY TELEPHONE:  (021)  689-5227  (RED CROSS POISON SERVICE)


TRANSPORT  EMERGENCY TELEPHONE:  (011) 331-2947  (HAZCHEM)





2.  COMPOSITION/INFORMATION ON  INGREDIENTS





CHEMICAL NAMES AND SYNONYMS:  PETROLEUM BITUMEN AND SOLVENT BLEND





INGREDIENTS CONSIDERED HAZARDOUS TO HEALTH:





Substance Name                                                           Wt%








OXIDIZED PETROLEUM ASPHALT                       58


(64742-93-40


HEAVY ALIPHATIC PETROLEUM                         28


SOLVENT (64742-96-7)


HEAVY AROMATIC PETROLEUM                         14


SOLVENT (64742-94-5)





See Section 15 for European label Information.





See Section 8 for exposure limits (if applicable).





3.  HAZARDS IDENTIFICATION





US OSHA HAZARD COMMUNICATION STANDARD:  Product assessed in accordance with OSHA 29 CFR 1910.1200  and determined to be hazardous.


EFFECTS OF OVEREXPOSURE:  Skin irritation can occur under conditions of poor personal hygiene.  Respiratory irritation, dizziness, nausea and even loss of consciousness can occur after excessive inhalation of vapours


EMERGENCY RESPONSE DATA:  Black Semi-solid liquid. Material is flammable DOT  ERG No.  -  128





See Section 11 for additional Biological hazards(if applicable).





4.  FIRST AID MEASURES





EYE CONTACT:  Flush thoroughly with water.  If irritation occurs, call a doctor.


SKIN CONTACT:  Wash contact areas with  suitable hand-cleaner.  Remove contaminated clothing.  Launder contamianted clothing before reuse.


INHALATION:  Remove from  further exposure.  If respiratory irritation, dizziness, nausea, or unconsciousness occurs, seek immediate medical assistance and call a doctor.  If breathing has stopped, use mouth to mouth resuscitation.


INGESTION:  Not expected to occur in normal  industrial use.  However do not induce vomiting.  Give 1 to 2 glasses of water.  Get medical assistance.


NOTE TO DOCTORS:  Material if aspirated into the lungs may cause chemical pneumonitis.  Treat appropriately.








5.  FIRE-FIGHTING MEASURES





EXTINGUISHING MEDIA:  Carbon dioxide, foam, dry chemical and water fog.


SPECIAL  FIRE FIGHTING  PROCEDURES:  Use water to keep fire exposed containers cool.  If a leak or spill has not ignited, use water to spray to disperse the vapours and to protect personnel attempting to stop leak.  Water spray may be used to flush spills away from exposures.  Prevent runoff from fire control or dilution from entering streams, sewers, or drinking water supply.


SPECIAL PROTECTIVE EQUIPMENT:  For fires in enclosed areas, fire fighters must use self-contained breathing apparatus.


UNUSUAL FIRE AND  EXPLOSION HAZARDS:  Material is flammable and if exposed to flames  can generate toxic fumes.


Flash Point C:  42 (ASTM D-93).  Flammable limits - LEL:  NE


UEL:  NE


NEPA HAZARD  ID:  Health:  0, Flammability:  2, Reactivity : 0


HAZARDOUS  DECOMPOSITION PRODUCTS:  Carbon monoxide.





6.  ACCIDENTAL RELEASE MEASURES





NOTIFICATION PROCEDURES:  Report spills as required to appropriate authorities.


PROCEDURES IF MATERIAL IS RELEASED OR SPILLED:  Adsorb on fire retardant treated sawdust, diatomaceous earth, etc.  Shovel  up and dispose of at an   appropriate waste disposal facility in accordance with current applicable laws and regulation, and product characteristics at time of disposal.


ENVIRONMENTAL PRECAUTIONS:  Prevent  spills  from entering storm sewers or drains and contact with soil.


PERSONAL PRECAUTIONS:  See Section 8





7.  HANDLING AND STORAGE





HANDLING:  Avoid contact with skin.  Trace amounts of Hydrogen Sulphide may be present.  Keep face clear of drum, tank and/or tank car opening.


STORAGE:  Store in a cool area.





8.  EXPOSURE CONTROLS/PERSONAL PROTECTION





VENTILATION:  Use in well ventilated area.  Ventilation desirable and equipment should be  explosion proof.


RESPIRATORY PROTECTION:  Approved respiratory protective equipment must be used when vapour or mist  concentrations exceed applicable standards.


EYE PROTECTION:  Normal industrial eye protection practices should be employed.


SKIN PROTECTION:  Impervious gloves should be worn.




















OCCUPATIONAL EXPOSURE LIMITS


                                                                                         ----LTEL-----                   ----STEL----             NOTE


Substance Name (CAS-No.)                         Source       ppm  mg/m3                  ppm  mg/m








OXIDIZED PETROLEUM ASPHALT


(64742-93-4) 


Fumes                                                        ACGIH                         5





HEAVY ALIPHATIC PETROLEUM        ACGIH         100       525


SOLVENT (64742-96-7)





HEAVY AROMATIC  PETROLEUM       ACGIH         100       525


SOLVENT (64742-94-5)                      





NOTE:  Limits shown for guidance only.  Follow applicable regulations.





THRESHOLD LIMIT VALUE:  100.00 ppm


                                                                                              


9.  PHYSICAL AND CHEMICAL PROPERTIES





Typical physical properties are given below.  Consult Product Data Sheet for specific details.





APPEARANCE:                                               Semi-solid Liquid


COLOUR:                                                         Black


ODOUR:                                                           Asphaltic


ODOUR THRESHOLD-ppm                             NA


pH:                                                                    NA


BOILING POINT C:                                         ( 190


MELTING  POINT C:                                      NA


FLASH POINT C:                                            42 (ASTM D-93)


FLAMMABILITY:                                           NA


AUTO FLAMMABILITY:                                NE


EXPLOSIVE PROPERTIES:                            NA


OXIDIZING  PROPERTIES:                            NA


VAPOUR PRESSURE-mmHg 20 C:                 ( 0.1


VAPOUR DENSITY:                                        NA


EVAPORATION RATE:                                  NA


RELATIVE DENSITY, 20 C, kg/1:                  0.923


SOLUBILITY IN WATER:                               Negligible


PARTITION COEFFICIENT:                           NE


VISCOSITY AT 60 C, cSt:                               60 - 110


VISCOSITY AT 100 C, cSt:                             NE


 POUR POINT C:                                             NA


FREEZING POINT C:                                      NE


VOLATILE  ORGANIC COMPOUND:           NE





NA=NOT APPLICABLE       NE=NOT ESTABLISHED       D=DECOMPOSES


FOR FURTHER TECHNICAL INFORMATION, CONTACT YOUR MARKETING REPRESENTATIVE 








10.  STABILITY AND REACTIVITY





STABILITY (THERMAL, LIGHT, ETC.):  Stable.


CONDITIONS TO AVOID:  Heat, sparks, flame and build up of static electricity.


INCOMPATIBILITY (MATERIALS TO AVOID):  Strong oxidizers.


HAZARDOUS DECOMPOSITION PRODUCTS:  Carbon monoxide.


HAZARDOUS POLYMERIZATION:  Will not occur.








11.  TOXICOLOGICAL DATA





-------ACUTE TOXICOLOGY------





ORAL TOXICITY (RATS):  Practically non-toxic (LD50:  greater than 2000 mg/kg).  ---Based on testing of similar products and/or the components.


DERMAL TOXICITY (RABBITS):  Practically  non-toxic (LD50:   greater than 2000mg/kg).  -----Based on testing of similar products and/or the components.


INHALATION TOXICITY (RATS):   Practically non-toxic (LC50:  greater than 5 mg/1).  ---Based on testing of similar products and/or the components.


EYE IRRITATION (RABBITS):  Practically non-irritating.  (Draize score:  greater than 6 but 15 or less).  ---Based on testing of similar products and/or the components.


SKIN IRRITATION (RABBITS):  Irritant.  (Primary Irritation Index:  3 or greater  but less than 5).  ---Based on testing of similar products and/or the components.


OTHER ACUTE TOXICITY DATA:  Inhalation of vapours/mists may cause respiratory system irritation.





--GENETIC TOXICOLOGY (SUMMARY)





Mutagenicity studies with similar materials in the ames, mouse lymphoma, rat bone marrow cytogenetics and dominant  lethal assays  showed no adverse effects.





-----REPRODUCTIVE TOXICOLOGY (SUMMARY)





Teratology studies with  similar materials were negative.





------NEUROTOXICOLOGY (SUMMARY)------





Animal  studies with similar materials by inhalation for 12 months showed  no significant neurotoxic, blood, kidney or other effects.








------CHRONIC TOXICOLOGY (SUMMARY)-------





This product contains solvents, prolonged repeated skin contact may defat the skin resulting in possible irritation and dermatitis.  Long term skin application studies with similar materials have resulted  in some systematic effects.  Chronic applications of similar materials to the skin of mice resulted in some skin tumours, due to extreme irritation to the  skin.





12. ECOLOGICAL INFORMATION





ENVIRONMENTAL FATE AND EFFECTS:


Acute LC/EC50 Fish:  Juvenile Rainbow Trout:  harmful ---Based on testing of similar products.











13.  DISPOSAL CONSIDERATIONS





WASTE DISPOSAL:  Product is suitable for burning in an enclosed, controlled burner for fuel value or disposal by supervised incineration.





RCRA INFORMATION:  The unused product, in our opinion, is not specifically listed by the EPA as a hazardous waste (40 CFR, Part 261D), nor is it formulated to contain materials which are listed  hazardous wastes.  It does not exhibit the hazardous characteristics of ignitability, corrosivity, or reactivity and is not formulated with contaminants as  determined by the Toxicity Characteristic Leaching Procedure (TCLP).


FLASH:  42 C











14.  TRANSPORT INFORMATION





USA DOT:


SHIPPING NAME:                                              Flammable Liquid, n.o.s. (contains ASPHALT,


                                                                            AROMATIC and  ALIPHATIC PETROLEUM 


                                                                            SOLVENTS)


HAZARD CLASS & DIV:                                   FLAMMABLE   LIQUID


ID NUMBER:                                                      NA1993


ERG NUMBER:                                                  128


PACKING GROUP:                                            PG III


STCC:                                                                 NE


DANGEROUS WHEN WET:                              NO


POISON:                                                             NO


LABEL(s):                                                           NA


PLACARD(S):                                                     FLAMMABLE


PRODUCT RQ:                                                   NA


MARPOL III STATUS:                                       NA





IMO:


HAZARD CLASS & DIV:                                    3.3


UN NUMBER:                                                     1993


PACKING GROUP:                                             PG III


SHIPPING NAME:                                              Flammable liquid, n.o.s.(contains ASPHALT 							        AROMATIC and ALIPHATIC PETROLEUM 


                                                                            SOLVENTS)


LABEL(s):                                                           Flammable liquid


MARPOL III STATUS:                                       NA





ICAO/IATA:


HAZARD CLASS & DIV:                                    3


ID/UN NUMBER:                                                1993


PACKING GROUP:                                             PG III


SHIPPING NAME:                                              Flammable liquids n.o.s. (contains ALPHALT 


                                                                            AROMATIC and ALIPHATIC PETROLEUM 


                                                                             SOLVENTS)


SUBSIDIARY RISK:                                           NA


LABEL(s):                                                           Flammable Liquid











15.  REGULATORY INFORMATION





Governmental Inventory Status:  All  components comply with TSCA, and EINECS/ELINCS.





EU Labelling:





	Symbol:  F Flammable.





Risk Phrase(s):  R10-20-38-52/53.


	Flammable.  Harmful by inhalation.  Irritating to skin.  Harmful to aquatic organisms, may cause 	long-term adverse effects in the aquatic environment.





Safety Phrase(s):  S36/37-43-61-62.


	Wear suitable protective clothing and gloves.  In case of fire use foam/drypowder/CO2/Halon.  	Avoid release to the environment.  Refer to special instructions/Safety data sheets.  If swallowed, 	do not induce vomiting:  seek medical advice immediately and show this container or label.





Contains:  Petroleum distillate.





U.S. Superfund Amendments and Reauthorization Act (SARA) Title III:


This product contains no “EXTREMELY HAZARDOUS  SUBSTANCES”.





SARA (311/312) REPORTABLE HAZARD CATEGORIES:


FIRE  CHRONIC  ACUTE





This product contains no chemicals reportable under SARA (313) Toxic Release Programme.





The following product ingredients are cited on the lists below:





CHEMICAL NAME                                                    CAS NUMBER                          LIST CITATIONS





HEAVY ALIPHATIC PETROLEUM SOLVENT          64742-96-7                                 1, 18, 19, 20, 21


                                                                                                                                         23, 25


HEAVY AROMATIC PETROLEUM SOLVENT          64742-94-5                                 1, 18, 19, 20, 23,


                                                                                                                                         25





----REGULATORY LISTS SEARCHED----


1=ACGIH  ALL          6=IARC  1              11=TSCA  4                17=CA  P65        22=MI  293


2=ACGIH  A1             7=IARC  2A           12=TSCA  5a2            18=CA  RTK      23=MN  RTK


3=ACGIH  A2             8=IARC  2B           13=TSCA  5e              19=FL  RTK       24=NJ  RTK


4=NTP  CARC            9=OSHA  CARC    14=TSCA   6               20=IL  RTK        25=PA  RTK


5=NTP  SUS               10=OSHA Z           15=TSCA   12b            21=LA  RTK      26RI  RTK





Code key:         CARC=Carcinogen;              SUS=Suspected Carcinogen





16.  OTHER INFORMATION





USE:  ADHESIVE, SEALANT AND WATERPROOFING AGENT.





INGREDIENT  DESCRIPTION                                         PERCENT                 CAS NUMBER








OXIDIZED PETROLEUM ASPHALT                                     58                           64742-93-4


HEAVY ALIPHATIC PETROLEUM SOLVENT                    28                           64742-96-7


HEAVY AROMATIC PETROLEUM  SOLVENT                  14                           64742-94-5








INFORMATION GIVEN HEREIN IS OFFERED IN GOOD FAITH AS ACCURATE, BUT WITHOUT GUARANTEE.  CONDITIONS OF USE AND SUITABILITY OF THE PRODUCT FOR PARTICULAR USES ARE BEYOND OUR CONTROL; ALL RISKS OF USE OF THE PRODUCT ARE THEREFORE ASSUMED BY THE USER AND WE EXPRESSLY DISCLAIM ALL WARRANTIES OF EVERY KIND AND NATURE, INCLUDING WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE IN RESPECT TO THE USE OR SUITABILITY OF THE PRODUCT.  NOTHING IS INTENDED AS A RECOMMEDATION FOR USES WHICH INFRINGE VALID PATENTS OR AS EXTENDING LICENSE UNDER VALID PATENTS.  APPROPRIATE WARNINGS AND SAFE HANDLING PROCEDURES SHOULD BE PROVIDED TO HANDLERS AND USERS.








Prepared for:  DALVEN PRODUCTS


                       P.O. BOX 371


                      EPPINDUST


	         7475


                     


For further information, contact :


Product Safety Adviser	(021) 403 4805		Cell :  083 628 4415





                                                                          


�PAGE  �








�PAGE  �6�


BITUSOL 4098











