MATERIAL SAFETY  DATA  SHEETS


(MSDS)





1.  PRODUCT AND COMPANY  IDENTIFICATION





Product Name:  Bitusol 7016	  	      	         	Updated:  9  September  1999


Supplier:  Dalven Products             		               	CAS Number:  Mixture  -  NA


                 P.O. Box 371                               	                          	UN Number:  UN1993


                 Eppindust


Health Emergency Telephone:  	(021) 689-5227   (Red  Cross Poison Service)


Transport Emergency Telephone:  	(011) 331-2947   (Hazchem)





2.  COMPOSITION/INFORMATION ON INGREDIENTS





Chemical names and synonyms: Petroleum Bitumen and  Solvent Blend  


Ingredients considered hazardous to health.





SUBSTANCE NAME	                                                                         	Wt%





OXIDIZED PETROLEUM ASPHALT                                                         	53


(64742-93-4)


UNOXIDIZED  PETROLEUM ASPHALT     	             		 	17


(8052-42-4)


LIGHT ALIPHATIC PETROLEUM                        	                        		15


SOLVENT (64742-89-8)


TOLUENE                                                                                                         	15


(108-88-3)


BENZENE (71-43-2)                                                                                        	( 0.01





See  section 15 for European label information


See section  B  for  exposure limits  ( if applicable).





3.  HAZARDS  IDENTIFICATION





US OSHA HAZARD COMMUNICATION STANDARD:  Product assessed in accordance with OSHA 29    CFR 1910.1200  and determined to be hazardous.


EFFECTS OF OVEREXPOSURE:  Skin iritation can occur under conditions of poor pesonal hygiene.  Respiratory irritation, dizziness, nausea and even loss of consciousness can occur after excessive inhalation of vapours.





EMERGENCY RESPONSE DATA:  Black  semi-solid  liquid..  Material is flammable


DOT ERG NO. 128


See  section  11 for additional Biological hazards (if applicable)





4.  FIRST AID MEASURES





EYE CONTACT:  Flush thoroughly with water.  If  irritation occurs, call a doctor.


SKIN CONTACT:  Wash contact areas with suitable hand -cleaner.  Remove contaminated clothing.  Launder contaminated clothing before use.


INHALATION:  Remove from further exposure.  If respiratory irriatation, dizziness, nausea, or unconsciousness occurs, seek immediate medical assistance and call a doctor.  If breathing has stopped, use mouth to mouth  resuscitation.


INGESTION:  Not expected to occur  in normal industrial use.  However do not induce vomiting.  Give 1 to 2 glasses of water.  Get medical assistance.


NOTE  TO  DOCTORS:  Material if aspirated into the lungs may cause chemical pneumonitis.  Treat appropriately.








5.  FIRE-FIGHTING MEASURES





EXTINGUISHING MEDIA:   Carbon dioxide, foam, dry chemical and water fog.


SPECIAL FIRE FIGHTING PROCEDURES:   Use water to keep fire exposed containers cool.  If a leak or spill has not ignited, use water  spray to disperse the vapours and to protect personnel attempting to stop leak.  Water spray may be used to flush spills away from exposures.  Prevent runoff from fire control or dilution from entering streams, sewers, or drinking water supply.


SPECIAL PROTECTIVE EQUIPMENT:  For fires in enclosed areas, fire fighters must use self-contained breathing apparatus.


UNUSUAL FIRE AND EXPLOSION HAZARDS:  Material is flammable and  if exposed to flames can generate toxic fumes.


Flash Point  C:  0 (ASTM D-92).                                                       Flammable limits-LEL:  NE


UEL:  NE





NEPA HAZARD ID:  Health:  0,   Flammability: 2,   Reactivity: 0


HAZARDOUS DECOMPOSITION  PRODUCTS:  Carbon monoxide.





6.  ACCIDENTAL RELEASE MEASURES





NOTIFICATION PROCEDURES:  Report spills as required to appropriate authorities.


PROCEDURES IF MATERIAL IS RELEASED OR SPILLED:  Adsorb on fire retardant treated  sawdust, diatomaceous earth, etc.  Shovel up and dispose of at an appropriate waste disposal facility in accordance with current applicable laws and regulations, and product characteristics at time of disposal.


ENVIROMENTAL PRECAUTIONS:  Prevent spills from entering storm sewers or drains and contact with soil.


PERSONAL PRECAUTIONS:  See section 8  





7.  HANDLING AND STORAGE





HANDLING:  Avoid  contact with  skin.  Trace amounts of Hydrogen Sulphide may be present.  Keep face clear of drum, tank and/or tank car opening.


STORAGE:  Store in a cool  area.





8.  EXPOSURE  CONTROLS/PERSONAL PROTECTION





VENTILATION:  Use in well ventilated area.  Ventilation desirable and equipment should be explosion proof.


RESPIRATORY PROTECTION:  Approved respiratory protective equipment must be used when vapour or mist concentrations exceed applicable standards..


EYE PROTECTION:  Normal industrial eye protection practices should be employed.


SKIN PROTECTION:  Impervious gloves should  be  worn.





OCCUPATIONAL EXPOSURE  LIMITS


	                                                                                 	LTEL          	STEL 	          NOTE


SUBSTANCE NAME (CAS-No)     	 SOURCE	ppm	mg/m3	ppm	mg/m3





OXIDIZED PETROLEUM ASPHALT


Fumes             (64742-93-4)                           ACGIH                    	    5


LIGHT ALIPHATIC PETROLEUM


SOLVENT (64742-89-8)                        	  ACGIH       	100 	 525


BENZENE (71-43-2)                                      OSHA                  1                          5


                                                                       ACGIH             	 10           32


TOLUENE (108-88-3)                                    OSHA  	100 	 375      	 150          560


Skin                                                                ACGIH	 50  	 188 











NOTE:  Limits  shown for guidance only.  Follow applicable regulations.





THRESHOLD LIMIT VALUE:  50.00ppm














9.  PHYSICAL AND CHEMICAL  PROPERTIES





Typical  physical  properties are given  below.  Consult product Data Sheet  for  specific details.





APPEARANCE: 		    	              Semi-solid Liquid


COLOUR:                                                    Black


ODOUR:                                                      Asphaltic


ODOUR THRESHOLD-ppm:                       NA


pH:                                                               NA


BOILING POINT C:                                    ( 190


MELTING POINT C:                                  NA


FLASH POINT C:                                       0 (ASTM D-92)


FLAMMABILITY:                                      NA


AUTO FLAMMABILITY:                          NE


EXPLOSIVE PROPERTIES:                       NA


OXIDIZING PROPERTIES:                        NA


VAPOUR PRESSURE-mmHg 20 C:            ( 0.1


VAPOUR DENSITY:                                  NA


EVAPORATION RATE:                             NA


RELATIVE DENSITY, 20 C, kg/1:             0.940


SOLUBILITY IN WATER:                         Negligible


PARTITION COEFFICIENT:                     NE


VISCOSITY AT 60 C, cSt:                          300 - 900


VISCOSITY AT 100 C, cSt:                       NE


POUR POINT C:                                        NA


FREEZING POINT C:                                NE


VOLATILE ORGANIC COMPOUND:      NE





NA=NOT  APPLICABLE         NE=NOT ESTABLISHED        D=DECOMPOSES





FOR  FURTHER TECHNICAL INFORMATION, CONTACT YOUR MARKETING REPRESENTATIVE.





10.  STABILITY AND REACTIVITY





STABILITY (THERMAL, LIGHT, ETC.) : Stable.


CONDITIONS TO AVOID:  Heat, sparks, flame and build up of static electricity.


INCOMPATIBITLITY (MATERIALS TO AVOID):  Strong oxidizers.


HAZARDOUS DECOMPOSITION PRODUCTS:  Carbon monoxide.


HAZARDOUS POLYMERIZATION:  Will not occur.





11.  TOXICOLOGICAL DATA


 ACUTE TOXICOLOGY


ORAL TOXICITY (RATS):  Practically non-toxic (LD50:  greater than 2000 mg/kg).  ------ Based  on testing of similar products and/or the components.


DERMAL TOXICITY (RABBITS):  Practically non-toxic (ID50:  greater than 2000 mg/kg).  ----Based on testing of similar products and/or the components.


INHALATION TOXICITY (RATS):  practically non-toxic (LC50)):   greater than 5 mg/1.  ----Based  on testing of similar products and/or the components.


EYE IRRITATION (RABBITS):  Practically non-irritating.  (Draize score: greater than 6 but 15 or less).  ---Based on testing of similar products and/or the components.


SKIN IRRITATION (RABBITS):  Irritant.  (Primary  Irritation  Index:  3 or  greater  but less than 5). -----Based on testing of similar products and/or the components.


OTHER ACUTE TOXICITY DATA:  Inhalation of vapours/mists may cause respiratory system irritation.





GENETIC TOXICOLOGY (SUMMARY)





Mutagencity studies with similar materials in the ames, mouse lymphoma, rat bone marrow cytogenetics and dominant lethal assays showed no adverse effects.














REPRODUCTIVE TOXICOLOGY (SUMMARY)





No birth defects were reported when pregnant rats, mice or rabbits were exposed to  vapours at levels up to 1000 ppm  of  toluene.  There were some toxic effects on developing  foetuses in other studies including  decreased body weight and survival at 267  ppm or greater.  A 2-generation reproduction study with rats did not result  in birth defects at levels up to 2000 ppm, but there was some growth inhibition.  The no-effect level (NOEL)  for toluene was 500





NEUROTOXICOLOGY (SUMMARY)





Toluene Inhalation studies (1500ppm), 6 hours/day, 5 days/week for 6 months in rats:  No neurotoxicity.





                                                 CHRONIC TOXICOLOGY (SUMMARY)





No carcinogenic effects were observed when toluene was applied to the skin of mice for two years.  A 2 year NTP inhalation study at levels up to 1200 ppm toluene showed no carcinogenic effects in rats or mice.  Prolonged or repeated exposure to toluene does not result in  bone marrow injury or blood changes characteristic of benzene.  Chronic exposure to high concentrations of benzene have been associated with blood abnormalities and leukemia in a  small  percentage of exposed workers.  NOTE  This product is specified to contain less than 0.01% m/m  benzene.





This product contains solvents, prolonged repeated  skin  contact may defat the skin resulting in possible irritation and dermatitis.





12.  ECOLOGICAL  INFORMATION





ENVIRONMENTAL  FATE AND EFFECTS:





Acute LC/EC50  Fish:  Juvenile Rainbow Trout:  Harmful -----Based on testing of similar products.





13.  DISPOSAL   CONSIDERATIONS





WASTE DISPOSAL:  Product is suitable for  burning in an enclosed, controlled burner for fuel value or disposal by supervised incineration.





RCRA INFORMATION:  The unused product, in our opinion, is not specifically listed by the EPA as a hazardous waste (40 CFR, Part 261D), nor is it formulated to contain materials which are listed hazardous wastes.  It does not exhibit the hazardous characteristics of ignitability, corrosivity,  or reactivity and is not formulated with contaminants as determined by the Toxicity Characteristic Leaching Procedure (TCLP). However, used product may be regulated.














14.  TRANSPORT INFORMATION





USA DOT:


	SHIPPING NAME: 		Flammable Liquid, n.o.s.(contains ASPHALT,


                                                                    	TOLUENE AND ALIPHATIC PETROLEUM 


                                                                    	SOLVENT)


	HAZARD  CLASS & DIV:	FLAMMABLE  LIQUID	


	ID NUMBER:   			NA1993


	ERG NUMBER:           		128


	PACKAGING  GROUP: 		PG II


	STCC:   			NE


	DANGEROUS WHEN WET: 	NO


	POISON:			NO


	LABEL(s):			NA


	PLACARD(s): 			Flammable


	PRODUCT RQ:			NA


	MARPOL  III STATUS:		NA





IMO:


	HAZARD CLASS & DIV:	3.2


	UN NUMBER: 			1993


	PACKAGING GROUP:		PG II


	SHIPPING NAME: 		Flammable Liquid, n..o.s. (contains ASPHALT


                                                                    TOLUENE  and ALIPHATIC PETROLEUM 


                                                                    SOLVENT)


	LABEL(s): 			Flammable Liquid


	MARPOL III STATUS: 		NA





ICAO/IATA:   


	HAZARD CLASS & DIV:  	3


	ID/UN  NUMBER: 		1993


	PACKING  GROUP: 		PG  II


	SHIPPING NAME:  		Flammable Liquid n.o.s. (contains  ASPHALT


                                                                    TOLUENE  and  ALIPHATIC PETROLEUM


                                                                    SOLVENT)


	SUBSIDIARY RISK: 		NA


	LABEL(s):			Flammable liquid





15.  REGULATORY INFORMATION





Governmental inventory Status:  All components comply with TSCA, AND EINECS/ELINCS.





EU  Labelling:


	Symbol:  F Flammable


	Risk Phrase(s):  R10-20-38-52/53


		Flammable.  Harmful  by inhalation.  Irritating to skin.  Harmful to  aquatic 			organisms,   may cause long-term adverse effects in the  aquatic enviroment.





	Safety Phrase(s):  S36/37-43-61-62


		Wear  suitable protective clothing and  gloves.  In case of fire use 				foam/drypowder/C02/HALON.  Avoid release to the enviroment.  Refer to special 		instructions/Safety data sheets.  If  swallowed,  do not induce vomiting:  seek 			medical advice immediately and show this container or label.





Contains:  Petroleum distillate, toluene and benzene ((0.1%)





U.S. Superfund Amendments and Reauthorization Act (SARA) Title III:  


This product contains no “EXTREMELY HAZARDOUS SUBSTANCES”.





SARA (311/312) REPORTABLE HAZARD CATEGORIES:


FIRE    CHRONIC    ACUTE





This  product contains the following SARA (313) Toxic Release Chemicals:





CHEMICAL NAME  			CAS NUMBER 			CONC.





BENZENE (COMPONENT ANALYSIS)   	71-43-2				(0.01%


TOLUENE                                                  	108-88-3			15%





U.S. Drug Enforcement Administration Chemical Diversion and  Trafficking Act:  Sales, receipts, movements or unexplainable losses of the following chemical(s) may require recordkeeping and reporting  in accordance with 21 CFR 1310/1313.   *** TOLUENE





The following product ingredients are cited on the lists below:





CHEMICAL NAME 				CAS NUMBER		LIST CITATIONS





LIGHT ALIPHATIC PETROLEUM SOLVENT      	64742-89-8            	1, 18, 19, 20, 21


                                                                                                                           23, 25


TOLUENE  (100.00%)				108-88-3		1, 10, 17, 18, 19,


                                                                                                                           20, 21, 22, 23, 24


BENZENE (COMPONENT ANALYSIS) 		71-43-2        		1, 3, 4, 6, 9, 17, 


 ((0.01%) 							          22, 24, 25								


REGULATORY LISTS SEARCHED





1: ACGIH ALL	  6:IARC    1             11:TSCA  4 		17:CA   P65	22:MI  293


2:ACGIH   A1    	  7:IARC    2 A         12:TSCA  5a2    	18:CA   RTK	23:MN RTK	


3:ACGIH  A2     	  8:IARC    2 B         13:TSCA   5e     	19:FL    RTK	24:NJ RTK


4:NTP  CARC    	  9:OSHA  CARC     14:TSCA   6       	20:IL     RTK	25:PA RK


5:NTP   SUS       	  10:OSHA   Z          15:TSCA  12b  	21:LA   RTK	26:RI RTK					


Code key:             CARC = Carcinogen;                        SUS = Suspected Carcinogen








16.  OTHER INFORMATION





USE:  ADHESIVE, SEALANT AND WATERPROOFING AGENT.





INGREDIENT DESCRIPTION			PERCENT                         CAS NUMBER





OXIDIZED PETROLEUM ASPHALT		    53                                  64742-93-4


UNOXIDIZED PETROLEUM ASPHALT                     17			8052-42-4


LIGHT ALIPHATIC PETROLEUM SOLVENT            15			64742-96-7


TOLUENE                                                                     15			108-88-3








INFORMATION GIVEN HEREIN IS OFFERED IN GOOD FAITH AS ACCURATE, BUT WITHOUT GUARANTEE.  CONDITIONS OF USE AND SUITABILITY OF THE PRODUCT FOR PARTICULAR USES ARE BEYOND OUR CONTROL; ALL RISKS OF USE OF THE  PRODUCT ARE  THEREFORE  ASSUMED BY THE USER AND WE EXPRESSLY  DISCLAIM ALL WARRANTIES OF EVERY KIND OF NATURE, INCLUDING WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE IN RESPECT TO THE USE OR SUITABILITY OF THE PRODUCT.  NOTHING IS INTENDED AS A RECOMMENDATION FOR USES WHICH  INFRINGE VALID PATENTS OR AS EXTENDING LICENSE UNDER VALID PATENTS.   APPROPRIATE WARNINGS AND SAFE HANDLING PROCEDURES SHOULD BE PROVIDED TO HANDLERS AND USERS.














Prepared for:	DALVEN PRODUCTS


		P. O. Box 371


		Eppindust   7475


For further information,  contact:	Product Safety Advisor	(021) 403 4805   (Cell)  083 628 4415











�PAGE  �








�PAGE  �1�


BITUSOL 7016











