PAGE  
1
BITUMEN ALUMINIUM  PAINT


MATERIAL SAFETY DATA SHEETS

(MSDS)

1.  PRODUCT AND  COMPANY IDENTIFICATION
Product Name:  Bitumen Aluminium Paint                                 Updated:  12 August 2005
Supplier:  DALVEN PRODUCTS                                               CAS  Number:  Mixture  -  NA

                     P.O. Box 371                                                                     

                    EPPINDUST

Health Emergency Telephone:  

(021)   658 5111  (Red Cross Poison Service)

Transport Emergency Telephone:  
(011)   975 1278/83  (Hazchem)

2.  COMPOSITION / INFORMATION ON INGREDIENTS

Chemical names and synonyms:  Petroleum  bitumen and solvent, and aluminium paste blend

Ingredients considered hazardous to health:

Substance Name



                         Approx Wt  %
OXIDIZED PETROLEUM ASPHALT                                         25
PETROLEUM SOLVENT




40
ALUMINIUM PASTE





25

See section 8 for exposure  limits (if applicable).

3.  HAZARDS   IDENTIFICATION

US OSHA HAZARD COMMUNICATION STANDARD:   Petroleum components have been assessed in accordance with OSHA 29 CFR 1910.1200 and determined to be hazardous.

EFFECTS OF OVEREXPOSURE:  Skin irritation can occur under conditions of  poor personal hygiene.  Respiratory irritation, dizziness, nausea and even loss of consciousness can occur after excessive inhalation of  vapours.

EMERGENCY RESPONSE DATA:  Silver low viscosity liquid.  Material is flammable.
See Section 11 for additional Biological hazards (if applicable)

4.    FIRST AID MEASURES

EYE CONTACT:  Flush thoroughly with water.  If irritation occurs, call a doctor.

SKIN CONTACT:  Wash contact areas with suitable hand-cleaner.  Remove contaminated clothing. Launder contaminated clothing before reuse.

INHALATION:  Remove  from  further exposure.  If respiratory irritation, dizziness, nausea, or unconsciousness occurs, seek immediate medical assistance and call a doctor.  If breathing has stopped, use mouth to mouth resuscitation.

INGESTION:  Not expected to occur in normal industrial use.  However do not  induce vomiting.  Give 1 to 2 glasses of  water.  Get medical assistance.

NOTE TO DOCTORS:  Material if aspirated into the lungs may  cause chemical pneumonitis.  Treat appropriately.

5.  FIRE-FIGHTING MEASURES

EXTINGUISHING MEDIA:  Carbon dioxide, foam, dry chemical and  water fog.

SPECIAL FIRE FIGHTING PROCEDURES:  Use water to keep fire exposed containers cool.  If a leak or spill has not ignited, use water spray to disperse the vapours and to protect personnel attempting to stop leak.  Water  spray may be used to  flush  spills away from  exposures.  Prevent runoff from fire control or dilution from entering streams, sewers, or drinking water supply.

SPECIAL PROTECTIVE EQUIPMENT:  For fires in enclosed areas, fire fighters must use self-contained breathing apparatus.

UNUSUAL FIRE AND EXPLOSION HAZARDS:  Material is flammable and if exposed to flames can generate  toxic  fumes.

Flash Point C:  27
HAZARDOUS  DECOMPOSITION PRODUCTS:  Carbon  monoxide.

6.  ACCIDENTAL  RELEASE MEASURES

NOTIFICATION PROCEDURES:  Report spills as required to appropriate authorities.

PROCEDURES IF MATERIAL IS RELEASED OR SPILLED:  Adsorb on fire retardant treated sawdust, diatomaceous earth, etc.  Shovel up and dispose of at an appropriate waste disposal facility in accordance with  current applicable laws and regulations, and product characteristics at time of disposal.

ENVIRONMENTAL  PRECAUTIONS:  Prevent spills from entering storm  sewers or drains and contact with soil.

PERSONAL PRECAUTIONS:  See section 8

7.  HANDLING AND STORAGE

HANDLING:  Avoid contact with skin.  Trace amounts of Hydrogen Sulphide may be present.  Keep face clear of drum, tank and /or  tank car opening.

STORAGE:  Store in  a  cool area.

8.  EXPOSURE CONTROLS / PERSONAL PROTECTION

VENTILATION:  use in  well ventilated area.  Ventilation desirable and equipment should be explosion proof.

RESPIRATORY PROTECTION:  Approved respiratory protective equipment must be used when vapour or mist concentrations exceed applicable standards.

EYE PROTECTION:  Normal industrial eye protection practices should be employed.

SKIN PROTECTION:  Impervious gloves should be worn.

9.  PHYSICAL AND CHEMICAL PROPERTIES

Typical  physical properties are given below.  
APPEARANCE                                                    Silver low viscosity liquid

COLOUR: 



          
Silver
ODOUR:                                                               Petroleum solvent
ODOUR THRESHOLD-ppm:                                   NA

pH:                                                                             NA

BOILING POINT C:                                                  NA
MELTING POINT C:                                                NA

FLASH POINT C:                                                     27 (ASTM D-93)

FLAMMABILITY:                                                    NA

AUTO FLAMMABILITY:                                        NE

EXPLOSIVE PROPERTIES:                                    NA

OXIDIZING PROPERTIES:                                     NA

VAPOUR PRESSURE-mmHg  20 C:                       ( 0.1

VAPOUR  DESNSITY:                                            NA

EVAPORATION RATE:                                           NA

RELATIVE DESNITY, 20  C, kg/1:                         0.96
SOLUBILITY IN WATER:                                       Negligible

PARTITION COEFFICIENT:                                   NE

VISCOSITY AT 60 C, cSt:                                       NE
VISCOSITY AT 100C, cSt:                                      NE

POUR POINT C:                                                       NA

FREEZING POINT C:                                              NE

VOLATILE ORGANIC COMPOUND:                   NE  

NA=NOT APPLICABLE         NE=NOT ESTABLISHED           D=DECOMPOSES

FOR FURTHER TECHNICAL INFORMATION, CONTACT YOUR MARKETING REPRESENTATIVE

10.  STABILITY AND REACTIVITY

STABILITY (Thermal, Light, ETC.):  Stable.

CONDITIONS TO AVOID:  Heat, sparks, flame and build up of static  electricity.

INCOMPATIBILITY (Materials to avoid):  Strong oxidizers.

HAZARDOUS DECOMPOSITION PRODUCTS:  Carbon monoxide.

HAZARDOUS POLYMERIZATION:  Will  not occur.

11.  TOXICOLOGICAL DATA 

--ACUTE TOXICOLOGY--

ORAL TOXICITY (RATS):  practically non-toxic - Based on testing of similar products and/or the components.

DERMAL TOXICITY (RABBITS):  Practically non-toxic - Based on testing of similar products and/or the components.

INHALATION TOXICITY (RATS):  Practically non-toxic - Based on testing of similar products and/or the components.

EYE IRRITATION (RABBITS):  Practically non-irritating. - Based on testing  on similar  products and/or the components.

SKIN IRRITATION (RABBITS):  IRRITANT. - Based on testing of similar products and/or  the components.

OTHER ACUTE TOXICITY DATA:    Inhalation of vapours/mists may cause respiratory system irritation.

----GENETIC TOXICOLOGY (SUMMARY)

Mutagenicity studies with similar materials in the ames, mouse lymphoma, rat bone marrow cytogenetics and dominant lethal assays showed no adverse effects.

--REPRODUCTIVE TOXICOLOGY (SUMMARY)--

Teratology  studies with similar materials were negative.

----NEUROTOXICOLOGY (SUMMARY)----

Animal studies with similar materials by inhalation for 12 months showed no  significant neurotoxic, blood, kidney or other effects.

--CHRONIC TOXICOLOGY (SUMMARY)---

This product contains solvents, prolonged repeated skin contact may defat the skin resulting in possible irritation and dermatitis.  Long term skin application studies with similar materials have resulted in some systemic effects.  Chronic applications of similar materials to the skin of mice resulted in some skin tumours, due to extreme irritation to the skin.

12.  ECOLOGICAL INFORMATION

ENVIRONMENTAL FATE AND EFFECTS:

Acute LC/EC50 Fish:  Juvenile Rainbow Trout:  Harmful---Based on Testing  .

13.  DISPOSAL CONSIDERATIONS

WASTE DISPOSAL:  product is suitable for burning in an enclosed area, or disposal by supervised incineration.

RCRA INFORMATION:  The unused product, in our opinion, is not specifically listed by the EPA as a hazardous waste(40 CFR, part 261D), nor is it formulated to contain materials which are listed hazardous wastes.  It  does not exhibit the hazardous characteristics of ignitability, corrosivity, or reactivity and is not formulated with  contaminants as determined by the Toxicity Characteristic leaching Procedure (TCLP).

FLASH:  27C

14.  TRANSPORT INFORMATION


SHIPPING NAME: 


Flammable liquid, n.o.s.(contains ASPHALT,

                                                                         
and ALIPHATIC PETROLEUM SOLVENT)


HAZARD CLASS & DIV:   

Flammable liquid


ID NUMBER:         


NA 1993


ERG NUMBER:  


128


PACKING  GROUP: 


PG III


STCC:  



NE


DANGEROUS WHEN WET: 

NO


POISON:  



NO


LABEL(s):



NA


PLACARD(s):



FLAMMABLE


PRODUCT RQ:


NA


MARPOL III STATUS:


NA

15.  REGULATORY INFORMATION

Symbol:  F Flammable.

Risk Phrase(s):  Flammable.  Harmful by inhalation.  Irritating to skin. Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

Safety phrase(s):  Wear suitable protective clothing and gloves.  In case of fire use foam/drypowder/C02/Halon.  Avoid release to the environment.  Refer to special instructions/Safety data sheets.  If swallowed, do not induce vomiting:  seek medical advice immediately and show this document.
Contains:  Petroleum distillate.

U.S. Super fund Amendments and Reauthorization Act (SARA) Title III:

This product contains no “EXTREMELY HAZARDOUS SUBSTANCES”.

SARA (311/312)  REPORTABLE HAZARD CATEGORIES:

FIRE  CHRONIC  ACUTE

This product contains no chemicals reportable under SARA (313) Toxic Release Programme.

The following product ingredients are cited on the lists below:

CHEMICAL NAME                                             CAS NUMBER                            LIST CITATIONS

HEAVY ALIPHATIC PETROLEUM SOLVENT    64742-96-7                         1, 18, 19, 20, 21

                                        






23, 25

LIGHT AROMATIC PETROLEUM SOLVENT     64742-95-6 

         18, 19, 20, 23, 2

                                                                                                                                    23, 25

---REGULATORY LISTS SEARCHED----

1=ACGIH  ALL               6=IARC  1               11=TSCA  4             17=  CA  P65           22=MI  293         
2=ACGIH  AI                  7=IARC  2A            12=TSCA  5a2          18=CA  RTK           23=MN  RTK

3=ACGIH  A2                 8=IARC  2B            13=TSCA  5e            19=FL  RTK             24=NJ  RTK 

4=NTP  CARC                9=OSHA  CARC     14=TSCA  6              20=IL   RTK             25=PA  RTK 

5=NTP  SUS                  10=OSHA Z              15=TSCA  12b          21=LA RTK             26=RI  RTK

Code key:          CARC=Carcinogen;         SUS=Suspected Carciogen

16.  OTHER INFORMATION

USE:  Paint intended to protect against the effects of ultraviolet.
INGREDIENT DESCRIPTION                                                    CAS NUMBER

OXIDIZED PETROLEUM ASPHALT                                                     64742-93-4

HEAVY ALIPHATIC PETROLEUM SOLVENT                                    64742-96-7

LIGHT AROMATIC PETROLEUM SOLVENT                                     64742-95-6

INFORMATION GIVEN HEREIN IS OFFERED IN GOOD FAITH AS ACCURATE, BUT WITHOUT GUARANTEE.  CONDITIONS OF USE AND SUITABILITY OF THE PRODUCT FOR PARTICULAR USES ARE BEYOND OUR CONTROL; ALL RISKS OF USE OF THE PRODUCT ARE THEREFORE ASSUMED BY THE USER AND WE EXPRESSLY DISCLAIM ALL WARRANTIES OF EVERY KIND AND NATURE, INCLUDING WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE IN RESPECT  TO  THE USE OR SUITABILITY OF THE PRODUCT.  NOTHING IS INTENDED AS A RECOMMENDATION FOR USES WHICH INFRINGE VALID PATENTS OR AS EXTENDING LICENSE UNDER VALID PATENTS.  APPROPRIATE WARNINGS AND SAFE HANDLING PROCEDURES SHOULD BE PROVIDED TO HANDLERS AND USERS.

Prepared for:  DALVEN PRODUCTS

                        P.O. BOX 371

                        EPPINDUST


           7475

For further information, contact:

Product Safety Adviser                (021) 593 8650                    Cell  082 807 0657
