PAGE  
1
FLEXICRETE


MATERIAL SAFETY DATA SHEETS

(MSDS)

1.  PRODUCT AND  COMPANY IDENTIFICATION
Product Name:  FLEXICRETE
                                 
Updated:  23 January 2004 
Supplier:  
DALVEN PRODUCTS                                    
                     
P.O. Box 371                                                                     

                    
EPPINDUST,


SOUTH AFRICA.

Tel.  
021 593 6790
Fax
021 593 8650.

HAZCHEM CODE :   2Z – NON-HAZARDOUS

Health Emergency Telephone:   
021  658 5111  (Red Cross Poison Service)

Transport Emergency Telephone:  011  975 1278/83  (Hazchem)

2.  COMPOSITION / INFORMATION ON INGREDIENTS

Styrene/butadiene co-polymer latex
3.  HAZARDS   IDENTIFICATION

Not classified as dangerous according to EC Directive 88/379/EEC and subsequent adaptations (CHIP 2 regulations in the UK)
4.  FIRST AID MEASURES

EYE CONTACT:  Flush thoroughly with water.  If irritation occurs, call a doctor.

SKIN CONTACT:   Skin irritation may be caused by prolonged contact. Wash contact areas with water and soap.  Remove contaminated clothing. Launder contaminated clothing before reuse.

INHALATION:  If dizziness or headache occur, remove  from  further exposure.  

INGESTION:  Not expected to occur in normal industrial use.  Get immediate medical assistance.

5.  FIRE-FIGHTING MEASURES

The product is an aqueous dispersion and is not combustible.
6.  ACCIDENTAL  RELEASE MEASURES

NOTIFICATION PROCEDURES:  Report spills as required to appropriate authorities.

PROCEDURES IF MATERIAL IS RELEASED OR SPILLED:  Adsorb on inert absorbent  sawdust, sand, diatomite, etc.  Shovel up and dispose of at an appropriate waste disposal facility in accordance with  current applicable laws and regulations, and product characteristics at time of disposal.

ENVIRONMENTAL  PRECAUTIONS:  Prevent spills from entering storm sewers or drains and contact with soil.

PERSONAL PRECAUTIONS:  See section 8

7.  HANDLING AND STORAGE

HANDLING:  No special precautions required with normal housekeeping. 
STORAGE:  
Protect from frost and direct sunlight. Temperature range +5ºC to +35ºC.
Do not store or transport in vessels or pipework made from aluminium, copper or its alloys.

Bulk tanks should be cleaned and sterilised at least annually to prevent accumulation of micro-organisms.
8.  EXPOSURE CONTROLS / PERSONAL PROTECTION

VENTILATION:  Use in  well ventilated area. When the product is dried or sprayed, local extraction is recommended.  
RESPIRATORY PROTECTION:  Suitable respiratory protective equipment must be used when ventilation is poor. Eg inside storage tanks or when spraying.
EYE PROTECTION:  Normal industrial eye protection practices should be employed.

SKIN PROTECTION:  Impervious gloves should be worn for frequent contact.
EXPOSURE CONTROLS: Contains low levels of volatile organic compounds which may evaporate during application and drying. Small quantities of ammonia need to be considered. 
9.  PHYSICAL AND CHEMICAL PROPERTIES

Typical  physical properties are given below.  Consult product data sheet for specific details.

APPEARANCE                                                         
Milky white liquid

ODOUR:                                                                    
Characteristic
pH:                                                                             
11
BOILING POINT C:                                                  
Approx. 100ºC (Water)
MELTING POINT C:                                                
N/A
FLASH POINT C:                                                     
N/A

FLAMMABILITY:                                                    
N/A

AUTO FLAMMABILITY:                                         
N/A
EXPLOSIVE PROPERTIES:                                    
N/A

OXIDIZING PROPERTIES:                                     
N/A

VAPOUR PRESSURE




Approx. 23hPa (Water) at 20ºC
RELATIVE DENSITY




Approx. 1.0
SOLUBILITY IN WATER:                                       
Insoluble but miscible in all proportions
PARTITION COEFFICIENT n Octanol/water                 
N/A
VISCOSITY – Brookfield 2/60 - 20ºC

         
100 cps
NA=NOT APPLICABLE         
10.  STABILITY AND REACTIVITY

STABILITY (Thermal, Light, ETC.):  Stable under recommended storage conditions.
11.  TOXICOLOGICAL DATA 

Long term experience indicates no danger to health when properly handled under industrial conditions.
12.  ECOLOGICAL INFORMATION

Not classified as dangerous to the environment according to directive 93/21/EEC
Behaviour in ecological compartment – Not applicable.

Additional advice – Long term experience with the dispersion when used as agreed, indicates no ecological damage.

13.  DISPOSAL CONSIDERATIONS

The product and waste containing the product should not be discharged directly into drains and waterways without treatment. The polymer content may be separated in a suitable coagulation and purification plant. Details available on request.
Disposal of the product, solid waste and packaging should always comply with local, national or EU regulations and be undertaken by an authorised contractor.

German waste code number :  Plastic dispersion : 57303

14.  TRANSPORT INFORMATION

Not classified as hazardous according to International Transport Regulations
15.  REGULATORY INFORMATION

Not classified as Dangerous according to EC Directive 88/379/EEC (including subsequent amendments) and requires no special labelling. 
16.  OTHER INFORMATION

This Material Safety Data Sheet conforms to EC-Directive 91/155 EEC and 93/112 EC
INFORMATION GIVEN HEREIN IS OFFERED IN GOOD FAITH AS ACCURATE, BUT WITHOUT GUARANTEE.  CONDITIONS OF USE AND SUITABILITY OF THE PRODUCT FOR PARTICULAR USES ARE BEYOND OUR CONTROL; ALL RISKS OF USE OF THE PRODUCT ARE THEREFORE ASSUMED BY THE USER AND WE EXPRESSLY DISCLAIM ALL WARRANTIES OF EVERY KIND AND NATURE, INCLUDING WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT  TO  THE USE OR SUITABILITY OF THE PRODUCT.  NOTHING IS INTENDED AS A RECOMMENDATION FOR USES WHICH INFRINGE VALID PATENTS OR AS EXTENDING LICENSE UNDER VALID PATENTS.  APPROPRIATE WARNINGS AND SAFE HANDLING PROCEDURES SHOULD BE PROVIDED TO HANDLERS AND USERS.
