DALVEN products

CAPE TOWN:   
     P.O. Box 371,  
Eppindust
 7475
Tel:  021 593 6790
Fax:  021 593 8650
JOHANNESBURG:      P.O. Box 1358,   
Florida   
 1710   
Tel:  011 472 4417   Fax:  011 472 1206
HEAD OFFICE:    
     P.O. Box 26738, Hout Bay 
 7872   
Tel:  021 790 2379   Fax:  021 790 3360

E-mail: info@dalven.co.za




    Website: www.dalven.co.za

ACRYLIC BITUMEN BLEND WATERPROOFING EMULSION

DESCRIPTION

ACRYLIC BITUMEN is a blend of bitumen emulsion and acrylic emulsion which when used in conjunction with a reinforcing fabric, produces a highly elastic film which is impervious to water.

TYPICAL USES

ACRYLIC BITUMEN, reinforced with an approved geotextile layer, is an excellent sealing compound, and is recommended for waterproofing of:


* Parapet walls


* Flashings


* Non-trafficable concrete and flat roofs


* Sealing of laps, screw holes and joints on corrugated roofs

SPECIAL CHARACTERISTICS

 * Highly resistant to degradation by UV rays

 * Flexible under all ambient temperatures

 * Easy to apply

 * Available in three colours (Grey, charcoal and red)

 * Thixotropic - will not flow on vertical walls

 * Resists fungus and mould growth

 * Non-toxic

 * All-in-one system - no primers or top coat required (except with metal which may require a rust inhibitor)

SURFACE PREPARATION

Remove all dust, dirt, flaking paint and laitence.  Moss and lichen should be removed and the surface treated with a suitable fungicide.  The fungicide should be allowed to dry before commencing the ACRYLIC BITUMEN treatment.

PRIMING

Non-metal surfaces

Prime the surface with a diluted coat of ACRYLIC BITUMEN (1 part water:1 part ACRYLIC BITUMEN).  

Metal surfaces

Prime with a suitable derusting agent to manufacturers specification.  New galvanised iron roofs should be degreased with an approved degreasing agent.

APPLICATION

1.  Apply the first coat at a rate of 1 litre per m2 and while still wet, imbed the geotextile membrane into this coat by means of a fluted roller, flat roller or stiff brush.

2.  Once dry, apply a saturation coat of ACRYLIC BITUMEN at approximately 0.7 litres per m2.

3.  Apply a final coat at approximately 0.5 litres per m2 at right angles to the saturation coat.

NOTES:

  i)  Do not apply if rain is imminent.  Ideally ACRYLIC BITUMEN should be applied in sunny weather.

 ii)  Do not apply to tar based materials unless they have weathered for two to three years.

iii)  Brushes or brooms should be washed in clean water immediately after use.  Once dry paraffin or white spirits may be needed.  (Do not use solvents during application).

SPECIFICATION

Product type:

Durable and elastic blend of acrylic and bitumen emulsions

Colour:


Charcoal, grey and red

Coverage:

2.2 litres per m2
Drying time:

2 - 3 hours out of doors

Flash point:

Non-flammable

Solids content:

50% m/m min

Viscosity:

110 - 130 Kreb units (soft paste)

Mass per litre:

1.1 - 1.2kg

Availability:

1 litre, 5 litre, 20 litre and 200 litre































DALVEN products cc;     Registration Number: 1997/ 004076/23;     VAT Number: 4720163015
Member: A.D.J. Swanepoel B.Sc.Eng., M.B.A.


